INSTALLATION & OPERATION MANUAL, TIPPER

Andy Brunskill

22/04/26

NEWTON TRAILERS AUTO SHEET

SYSTEM

www.newtontrailers.com

Newton Trailers Limited

Station Yard, Station Road

Ridgmont, Bedfordshire

MK43 0XP

01525 872466

Table of Contents

Introduction .................................................................................

Warranty ................................................................................................................ i4

Safety ..................................................................................................................... i5

Safety Icons………………………………………………………………….…………………………………i5

Fitting

Tools Required ......................................................................................................... 2

Fasteners ................................................................................................................ 2

Kit contents ............................................................................................................. 3

System Layout ......................................................................................................... 4

Wear Plates ............................................................................................................. 7

Hose Position .......................................................................................................... 8

Wiring ................................................................................................................... 10

Centre Motor ......................................................................................................... 12

Strap Fitting ........................................................................................................... 14

Front position ..................................................................................................... 14

Middle position................................................................................................... 14

Rear position ...................................................................................................... 14

Pole roller fastening ............................................................................................ 14

Bolting the straps to the pulley ............................................................................. 15

Fitting the pulley cover ........................................................................................ 16

Strap clamping ................................................................................................... 17

Storm straps ...................................................................................................... 18

Securing the sheet poles ..................................................................................... 20

Final system checks ............................................................................................... 21

Trouble shooting .................................................................................................... 22

Schematics ……………………………………………………………………………………………………23

Newton Trailers Limited

Introduction

It is strongly recommended that installers and operators read this manual in its entirety

before beginning the installation or operation of the system.

WARNING: Always disconnect electrical power to the trailer before entering the trailer or

working on any of the Auto sheet Tipper System components. Failure to do so may result

in serious injury or death due to the voltage involved in the sheeting system.

IMPORTANT: Installing the Auto sheet Tipper System requires some alterations to your

trailer. Changes made without approval from the trailer manufacturer may void trailers

warranty.

Newton Trailers Limited

This manual explains procedures for installing and operating the Newton Trailers Auto sheet Tipper

System. Many variables affect the installation, but the general process remains consistent. Details of

the installation can vary according to trailer features and installer preferences.

Warranty

This Auto sheet System is warranted to the original purchaser to be free from defects in

material and workmanship under normal use for a period of one year from the date of

purchase. During the warranty period, and upon proof of

purchase, the Auto sheet System will be repaired or replaced with the same or similar model.

This warranty shall not apply to any parts that; (a} have been repaired or altered

outside of the Auto sheet System; (b) have been subjected to misuse, negligence

or accident; or (c) have been used or installed in a manner contrary to these Auto

sheet System instructions.

In certain circumstances some states do not allow the exclusion or limitation of

incidental damage, some or all the above limitations or exclusions may not apply

to you. This warranty gives you specific legal rights, and you may have other rights

that vary from country to country.

If this warranty violates law: To the extent any provision of this warranty contravenes

the law of any jurisdiction, that provision shall be inapplicable in such jurisdiction

and the remainder of the warranty shall not be affected thereby.

Notice: To validate all warranties, a proof of purchase with date and purchaser along

with a brief write up of any faults with the items returned, must be supplied. Failure

to do so will result in nonpayment and warranty items being scrapped.

Newton Trailers Limited

IT IS EXPRESSLY AGREED THAT THIS SHALL BE THE SOLE AND EXCLUSIVE

REMEDY OF THE BUYER. UNDER NO CIRCUMSTANCES SHALL NEWTON TRAILERS BE

LIABLE FOR ANY COSTS, LOSS, EXPENSE, DAMAGE, SPECIAL DAMAGES,

INCIDENTAL DAMAGES, OR CONSEQUENTIAL DAMAGES ARISING

DIRECTLY OR INDIRECTLY FROM THE USE OF THE CLEAN SWEEP SYSTEM,

WHETHER BASED UPON WARRANTY, CONTRACT NEGLIGENCE OR STRICT LIABILITY.

THE WARRANTY AND LIMITS OF LIABILITY CONTAINED HEREIN ARE IN

PLACE OF ALL OTHER WARRANTIES AND LIABILITIES, EXPRESSED OR

IMPLIED. ALL IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS

FOR A PARTICULAR PURPOSE ARE HEREBY DISCLAIMED BY NEWTON TRAILERS AND

EXCLUDED FROM THIS WARRANTY.

Safety

FAILURE TO FOLLOW SAFETY GUIDELINES MAY RESULT IN INJURY OR DEATH.

Newton Trailers Limited

Your safety is of the utmost importance to Newton Trailers. Follow safe working practices while installing this

system. Review all safety guidelines and pay attention to safety warning symbols as they will highlight specific

dangerous aspects of the task at hand.

Only properly trained and authorised personnel shall install, operate and service Newton Trailers equipment.

Installers are expected to have a fundamental understanding of tool safety and safe working practices.

DO NOT deviate from these instructions or modify the equipment. Modifications can render the equipment unsafe

and void your warranty.

Newton Trailers is not responsible for equipment damage or personal injury caused by failure to follow these

instructions or to incorporate safe working practices.

The following safety symbols are potential hazards of installing/using this equipment, mandatory

notices and safety precautions that should be taken while working with Newton Trailers equipment. A

description of the alert will accompany each icon. Familiarize yourself with them below to quickly

identify important information while working on this equipment.

Tools Required

Metric socket set

Metric spanner set

Side cuts

Drill bits 4.8, 5, 6.8, 10, 12, 13 & 16mm

Drill taps 8 & 10mm

Rivet gun

Nut insert tool

Grinder where required

Tape measure

Hammer

Gas torch

Electrical crimps 25mm

Fasteners

It is recommended,

• Where possible to use stainless steel fasteners to reduce corrosion.

• Where possible use hex button socket bolts.

• Copper grease must be used to prevent pick up on bolt threads.

• Copper grease must be used to prevent thread pick up on drill bits & taps used.

Newton Trailers Limited

Kit contents

Part No.

Description

Quantity

SK-PIAB

24Vdc HPU ASSEMBLY

1

SK-SMA

STRAP MOTOR ASSEMBLY

1

SK-CPDA-V2

TOP MOTOR ASSEMBLY

1

HC-POLESTOP

SHEET POLE STOP

3

HC-1PSG

1 PORT STRAP RUNNER

3

HC-2PSG

2 PORT STRAP RUNNER

2

SK-TIP2STRAP

TIPPER STRAP SET

2

HC-CLAMPASSYS

STRAP CLAMPS

3

HC-SIDEPOLEROLLER

NYLON POLE ROLLER

2

HC-BSL-A

COILED ANDERSON LEAD

1

BK-175A

ANDERSON PLUG

1

SK-TIPANDERSON

ANDERSON LEAD KIT

1

HC-APP

ANDERSON PLUG PLATE

1

HC-THB

TOP HOSE BRACKET

1

HK-ADKIT

HYDRAULIC ADAPTOR KIT

1

1/4BULKHEAD

¼ BULKHEAD FITTINGS

2

HK-STA

HYDRAULIC HOSE KIT

1

HC-TSP

TOGGLE SWITCH PLATE

1

HC-SRP

STRAP RUNNER PLATE

4

SK-STABK

STAS BOLT KIT

1

HC-FRWP

FRONT & REAR WEAR PLATE

2

HC-TIPWPC

TIP WEAR PLATE CENTRE

1

Newton Trailers Limited

System Layout

Newton Trailers Limited

Step 1: Mount the powerpack box bracket, The most common mou

position of the powerpack box is at the top righthand side, 457mm d

uppermost point of the bulkhead, Fix using 10mm bolts with locking nuts (torque to

fastener manufacturers spec).

Step 2: Mount the brake motor backing plate assembly 457mm down from the upper

most point of the bulkhead ﬁx using 10mm bolts with locking nuts (torque to fastener

manufacturers spec), (plate the inner wall of the extruded material to prevent crushing),

Note: Box and motor bracket mounts not required for square bodies.

Step 3: Mount the powerpack box to the box bracket (if used), use the rear of the

powerpack box as a template and mark 4 holes and drill, ﬁt M12 coach bolts and ﬁx the

nut on the bolt and slot the powerpack over the head of the bolts into the slots, hang the

box and tighten the locking nuts (torque to fastener manufacturers spec).

Step 4: Mount the brake motor bracket to the Alu backing plate (if used) at the bottom

left-hand side, drill 4 holes centrally in the bolt slots to allow adjustment up and down,

ﬁx using M12 coach bolts with locking nuts (torque to fastener manufacturers spec).

Step 5: Mount the brake motor with M10x50 coach bolts and locking nuts (torque to

fastener manufacturers spec), Fit the pulley to the motor, to make future servicing easy

only tighten the pulley retaining bolt ﬁnger tight.

Step 6: Mount the ﬁrst 2 port strap guide using the pulley position as a template, bolt

through the Alu brake motor assembly using M8x70 bolts with locking nuts (torque to

fastener manufacturers spec).

Step 7: Fitting HC-SRP strap mounting plates to the sidewall. Using the strap mounting

plate as a template offer the plate to the body with the top of the plate 489mm down

from the top of the chop rail, mark the 4 outer holes in the 4 remaining strap guide

positions. Drill holes through the outer skin of the sidewall and ﬁx in each hole an 8mm

nut-sert, follow on for all strap positions and ﬁx to the sidewall using M8x30 button

bolts, 4 per plate (torque to fastener manufacturers spec).

Newton Trailers Limited

Step 8: Drill through the strap guide plate and outer skin of the body

appropriate hole for the strap guide position using an M8 drill tap.

For example,

1st intermediate – drill tap top 2 holes.

Middle – top 3 holes.

2nd intermediate - middle two holes.

Rear – Bottom two holes.

This is so that the straps run in line and is more aesthetically pleasing on the eye.

Step 9: Mount the Strap guides to the strap guide plates using M8x70 with locking nuts

(torque to fastener manufacturers spec).

Newton Trailers Limited

Wear Plates

Step 1: If dome wear strips are to be ﬁtted, lay out the nylon strips on the domes and

drill 5mm holes through the nylon and the dome, equal spacing 300mm apart length

ways and across the width. Fasten using 4.8mm large ﬂange rivets.

Front dome

The orientation of the plates must be noted, the side of the plate with 5 holes ﬁxes to the

body, the bottom of the plate should be ﬂush with the bottom of the top rail proﬁle.

Offer up the wear plate as above and mark the line to be folded.

Attach the plate with 4.8mm large ﬂange rivets on the side and 6.8 mono bolts on the

top.

Rear dome

Repeat the process as above.

Centre hoop position

When the double hoop is ﬁtted (as in the picture above) the process is repeated as with

the front & rear wear plates. Apart from the horizontal plane. Fix using 6.8mm mono-

bolts and shape the wear plate around the hoops with a hammer to eliminate and

contact with the sheet.

NOTE: To reduce any contact between the sheet and centre hoops on the nearside a

HC-WEARSTRIP may be required, this can be fastened into the centre hoops using

4.8mm large ﬂange rivets.

Newton Trailers Limited

Hose Position

Step 1: Fit the HC-THB 600mm from the left-hand side of the trailer below the dome,

using the hose bracket as a template mark and using the M8 drill tap, tap the two

mounting hose and ﬁx using M8x16 bolts (torque to fastener manufacturers spec).

Step 2: Fit ¼” bulkhead

ﬁttings with nuts for the pipes

to attach.

Step 3: Fit the hoses from the kit (variable length) from the right-hand side pressure port

on the powerpack box, to the top hose bracket. From the top port on the box to top of

the bracket. From the bottom port on the box, to bottom on the bracket, leaving the

hoses loose on the hose tails for free movement with securing.

Step 4: Secure one hose with a 16mm hose clip using 4.8mm self-drilling Torx screws,

equal spacing. Cable tie the remaining hose to the ﬁxed hose and trim ﬂush with a

Stanley knife.

Newton Trailers Limited

Step 5: Fit the hoses for the ZBMR brake motor. On the left-hand side of the box. Top

port to middle on the brake motor, Middle port on the box to top on the brake motor,

bottom port on the box to the bottom port on the brake motor (use hose adaptors if

required).

Note: Check and tighten all hose ﬁttings, double check preassembled components.

Adaptors required:

½” x ¼” M/M x2

¼” x ¼” M/M x1

½” bonded washer x2

Remove the upper blanking plug on the bottom of the motor (H6) and re use the ¼”

bonded seal for the pilot line.

Newton Trailers Limited

Wiring

Step 1: Mount the HC-TSP, Using the toggle switch plate as

a template mark the two mounting hole and drill using the

M8 drill tap. Fix using M8x16 stainless steel fasteners

(torque to fastener manufacturers spec).

Step 2: Mount the toggle switch, run the manual over-ride switch cable through the

toggle switch plate and bolt using M5x25 machine screws and locking nuts (torque to

fastener manufacturers spec). Run the cable up into the powerpack box and plug into

the PCB board of the sheet remote kit, leave the excess cable hanging loose for step 9.

Step 3: Run the battery cable through the conduit.

Step 4: Starting from your Anderson plug mounting position run the cable the length of

the chassis and up the body in line with the cable entry hole in the bottom of the

powerpack box. Secure the Anderson lead to the chassis loom using 7.6mm cable ties

and run up the body bridging the cross member free from any pinch points, secure with

30mm p-clips (torque to fastener manufacturers spec).

Step 5: Mount the HC-APP, using the Anderson plug plate as a template mark the holes

in the required position and drill to 6mm, Fix using M6x25 bolts and locking nuts (torque

to fastener manufacturers spec).

Step 6: On the Anderson plug end strip the cable back 20mm and solder on the

connection pins, heat shrink over the female side of the connector pin onto the cable to

prevent water ingress.

Step 7: Push the pins into the Anderson plug and mount to the Anderson plug mounting

plate using M6x40 bolts and locking nuts (torque to fastener manufacturers spec).

Step 8: Fasten the Anderson lead up the front bulkhead using 30mm P-clips and (torque

to fastener manufacturers spec).

Newton Trailers Limited

Step 9: Secure the manual over-ride switch to the Anderson lead using 4.8mm cable

ties and cut ﬂush to prevent sharp edges.

Newton Trailers Limited

Centre Motor

Note: The ﬁtting of the top motor can be done if the sheet & poles are being ﬁt as new,

sheet ﬁtting manual is a separate document.

Roll the sheet all the way over to the closed position to carry out the next steps.

Step 1: Fitting the top motor assembly,

start by inserting the nylon sleeve into the

centre sheet pole, Insert the remainder of

the assembly into the nylon sleeve

leaving the tarp tube drive sticking out of

the nylon sleeve by approx. 5mm.

Step 2: (Option A) Using a 10mm bolt and a pair of mole grips clamped on the bolt head

heat up the bolt thread until glowing and burn through the sheet reinforcement webbing

in 3 positions (see picture below), Using those 3 positions as reference drill-tap to

10mm through the pole and tarp tube drive. Fix using M10x30 button hex bolts & repair

washers (torque to fastener manufacturers spec).

Step 3: (Option B) Using an 8mm bolt repeat the process above but drill through the

pole and tarp tube drive to 6.8mm and ﬁx using 6.8x25mm Mono-bolts and repair

washers.

Newton Trailers Limited

Step 4: If required trim the nylon roller axle ﬂush with the nut. Depending on the sheet

ﬁtting, the nylon roller axle may need to be cut ﬂush with the nut to prevent contact with

the side pole when the sheet is in operation.

Step 6: Fit the top motor hoses to the motor and HC-THB bracket, Top port on the

bracket to top port on the motor and bottom to bottom port.

Adaptors required:

½” x ¼” M/M x2

½” bonded seals x2

Newton Trailers Limited

Strap Fitting

Front position

Step 1: Offer up the strap to the side pole at 90 degrees, mark the pocket to be cut

before cutting out with a Stanley knife. If pole rollers are to be ﬁt, elongate the pocket to

be cut out.

Step 2: If a pole roller is to be ﬁt slide the side sheet pole to the rear, insert the pole

roller into the pocket and slide the pole back making sure the pole roller slides freely

over the side pole.

Step 3: Using the blow torch, burn off any loose stitching post pocket cutting.

Middle position

Repeat step 1 as above, pole rollers are not ﬁtted to the middle of the trailer.

Rear position

Repeat step 1 as the front position and step 2 if a pole roller is to be ﬁtted.

Pole roller fastening

If pole rollers are to be ﬁtted, follow the next step.

Mark either side of the pole roller, ensuring the roller runs across the domes. Using the

M8 drill-tap drill, drill through to the centre of the pole and fasten using M8x16 button

headed bolts (torque to fastener manufacturers spec).

Repeat for the rear pole roller.

HC-SIDE POLE CLAMP

Sold separately

Newton Trailers Limited

Pole rollers are ﬁt to reduce the side pole encountering the body, wh

roller is ﬁt to the side pole, the sheet strap encounters the side of th

increasing wear on both the sheet and strap. The most common place a sheet strap

breaks is on the rear of the pole when the strap is clamped.

When the pole rollers are coupled with the alu torque arm with roller this reduces wear

on the sheet and straps and removes the need for the nylon dome wear strips HC-DWS.

Bolting the straps to the pulley

Step 1: Thread the 5.05m front strap from the SK-TIP2STRAP kit through the ﬁrst strap

guide.

Step 2: Apply Vaseline to the threads of the strap retaining bolt.

Step 3: Insert the eye of the short strap into the top recess of the pulley and slide in the

M6x90 Cap head bolt to retain the front strap,

Step 4: Thread the longer split section of the SK-TIP2STRAP kit through the strap guides

from the front. The shorter stitched section feed in the top port of the middle strap guide

and the longer section through the bottom port and onto the rear.

Step 5: Insert the eye of the longer strap into the bottom recess of the pulley and thread

the bolt through the strap eye and into the thread of the pulley, tighten ﬁnger tight only.

This makes future strap replacement easier.

Note: It is important to make sure all straps are not twisted or tangled.

Step 6: Wrap the straps around the pulley anticlockwise so that when the strap is

unwinding it runs clockwise and releases from the spool to the outside of the body.

Wind the straps up until the stitched section of the middle and rear strap is just about to

enter the front strap guide. Pull the straps tight on the pulley and repeat.

Newton Trailers Limited

Step 7: Spray the ZBMR brake motor shaft with copper grease or us

Fit the pulley to the ZBMR brake motor and fasten with an M8x50 bol

ﬂat washer, tighten using a hand ratchet and nip the bolt.

Note: Do not over tighten as this will make strap replacement and splitting the pulley

from the motor easier.

Fitting the pulley cover

Step 1: Mark two holes on the mounting ﬂange of the pulley cover.

A: If pulley cover is plastic offer up over the pulley and drill the marked holes with a

4mm drill bit, Fix using 4.8 x 25mm dome headed torx self-drilling screws to prevent the

plastic from splitting (torque to fastener manufacturers spec).

B: If pulley cover is Aluminium then drill the marked holes to 10mm using a10mm drill

bit, Mark the ﬁxing holes on the Aluminium brake motor body bracket using an 8mm

drill-tap. Fix using M8x16mm dome headed hex bolts and washers (torque to fastener

manufacturers spec).

Note: Make sure the pulley cover does not encounter the pully or straps when mounted.

Newton Trailers Limited

Strap clamping

Step 1: Wrap the strap around the pole from the rear and ﬁt the strap clamp with the

studs facing away from the body as in the above image. Pull the strap tight and ﬁt the

ﬁrst clamp plate, run the strap back up over the outer side of the plate and ﬁt another

plate over the top and ﬁt the locking nuts (torque to fastener manufacturers spec).

Never over tighten as the studs can spin where they are spot welded to the rear plate

and the threads can strip; it is recommended that a hand ratchet be used for these.

Step 2: Cut the strap to length, burn of the end to prevent fraying.

Step 3: Cable tie the cut off end to the bottom side of the strap clamp and cut the tail of

the cable tie ﬂush.

Repeat for all steps for all 3 strap positions.

Newton Trailers Limited

Storm straps

Note: If storm straps are not required skip this section.

Fitting the storm strap eye

Step 1: Mark the desired position on the side pole for a pocket to be cut for the HC-

STORMSTRAPEYE (bearing in mind the side wall construction). Cut the sheet and burn

off and threads remaining.

Step 2: Wrap the storm strap eye around the pole from the rear so the eye is hanging

away from the body, temporarily secure the storm strap eye in position with duct tape.

Step 3: heat up a 10mm bolt with the blow torch until glowing and burn a hole through

the strap either on the underside of the pole or outer edge.

Step 4: Once the molten webbing has cooled, drill tap an 8mm hole through to the

centre of the pole and secure using an M8x30 button headed bolt & repair washer

(torque to fastener manufacturers spec).

Note: Alternatively, this can be drilled to 6.8mm and a mono-bolt with washer can be

used.

Newton Trailers Limited

Fitting the storm strap anchor

Step 1: Offer the storm strap anchor 100mm below the storm strap eye (bearing in mind

the side wall construction).

Step 2: Drill through the body with a 10mm drill bit using M10x50 bolts with locking

nuts, repair washers or a backing plate may be required to prevent crushing, (torque to

fastener manufacturers spec).

Step 3: Hang the storm strap from the anchor to line up for the HC-SSG ﬁtting. Place the

storm strap guide on the bottom rail of the body, mark the mounting holes and drill to

5mm, ﬁx using 4.8mm rivets.

Fixing the storm strap ratchet

Step 1: Finding the desired position for the HC-SSR, position the storm strap ratchet in

the desired position and mark a hole to secure the ratchet. Drill to either 8 or 10mm and

secure with bolts & locking nuts.

Note: If the ratchet is binding in the mounting position, pack out between the body and

the ratchet with washers or weld a cut section of 50x75mm Alu or Steel box section.

Step 2: Cut the storm strap tail to length with a diagonal cut and burn off the tail to

prevent fraying.

Step 3: Lubricate the ratchet.

Note: Cutting the strap tail diagonally aids threading the strap into the ratchet.

Newton Trailers Limited

Securing the sheet poles

If the centre pole has been secure in the sheet ﬁtting section (separate manual) then

miss this next section.

Centre pole

Step 1: Using half round 25mm beading cut four pieces into 350mm lengths and lay out

on the top of the sheet pole, measurements are variable as long as its equal distances.

First position can be 100mm from the webbing strip sewn into the sheet, mirrored on

the rear then either side of the centre hoop for the middle.

Step 2: Drill x3 5mm holes in each strip 30mm in from each end one central, fasten

using 4.8 x 25mm large ﬂange rivets.

Step 3: Drill to 5mm and rivet the webbing strip at the rear of the centre sheet pole and

fasten with 4.8 x 25mm large ﬂange rivets.

Step 4: Fit RTI1164 sheet pole end cap if required.

RTI1164 – 88mm plastic insert – centre pole

Side pole

Step 1: Marking the ﬁxing points to secure the side pole in the pole pocket. Mark a point

on the front sheet webbing strip, either side of the sheet strap pocket, middle either side

of the strap pocket and rear either side of the strap pocket.

Step 2: Drill holes to 5mm and ﬁx using 4.8mm x 25 large ﬂange rivets.

Step 3: Fit RTI455 side pole end caps if required.

Note: Side pole end caps can be sold separately to close off the open ends of the sheet

pole.

RTI455 – 50mm plastic insert – side pole

Newton Trailers Limited

Final system checks

Step 1: Fill the sheet pack with oil.

Note: It is recommended that ATF ﬂuid be used to prevent bacterial growth clogging up

the tank suction ﬁlter.

Step 2: Check all hydraulic ﬁttings are tight with a spanner to prevent any oil leakage.

Step 3: Turn on the Icarus remote set and check power status, Pairing the remote

system to the Icarus app can run ﬁrmware updates and check for any electrical issues.

Step 4: Check all persons are clear of the sheet before operating.

Step 5: Operate the sheeting system open and closed 4 or 5 times.

Step 6: Check for oil leaks.

Step 7: Top up oil if required.

Step 8: Check sheet straps for tightness and adjust if required.

Note: It is important that the oil level and all ﬁxing and hoses be checked 1 week after

system ﬁtting and that the sheeting system must be inspected upon the date over every

vehicle inspection.

Max sheet pack operating pressure 150 Bar when sheet hits the sheet stops or the limit

when closing.

Optimum pressure with Aluminium torque arm and side pole rollers the sheet system

pressure runs at 50 bar, when the sheet hits the apex of the dome the pressure will spike

to 90 Bar.

ZBMR brake pilot pressure to release 25 BAR.

Never:

Under any circumstance must the pressure relief valve be adjusted, this is preset at 150

Bar, Increasing the pressure relief can burst motor seals, split the roof sheet and pull

the sheet strap clamps loose.

Newton Trailers Limited

Trouble shooting

• If the Icarus remote turns on but does not pair, Check the status light on the

receiver.

• No status light on the receiver? Check power to the sheet pack starter using a

test lamp or multi meter.

• No power to starter motor? Check Anderson plug connections and power supply

form source.

• Icarus remote paired to receiver but no output action? Pair the receiver to the

Icarus app and check error codes and ﬁx.

• No error codes from the Icarus system? Check output from the starter motor and

replace if needed.

• Starter motor working but no movement in hydraulic system? Check pressure

output on the pressure gauge.

• No pressure at the gauge? Check oil level.

• Starter motor working and pump starting no output? Check power to coils & test

coils.

• Sheet motor struggling to open/close sheet? This indicates a weak motor.

Fit adaptor pressure gauge adaptor in pressure lines and check pressures to

motors. Check the ZBMR pilot line for pressure to release the brake (refer to

system checks for pressures).

Icarus operating and troubleshooting manual comes with the remote set, further

digital copies available upon request.

Newton Trailers Limited

Newton Trailers Limited

Newton Trailers Limited

Newton Trailers Limited

Newton Trailers Limited

Newton Trailers Limited

Newton Trailers Limited

Station Yard, Station Road

Ridgmont, Bedfordshire

MK43 0XP

01525 872466

sales@newtontrailers.com